It’s very good that you sended DSN, because I thought you didn't change the schematic.

But you changed DSN and
I’m sorry, but there is error in schematic DDR.

Engineer changed DSN, swap bits inside every Byte.

But he shouldn’t to change first bit of every bite, it’s mistake.

In Routing Guidelines that I sended to you, was that:

• Exchange of pins and gates

– Pin Swapping: Data bits (DQ[31:0]) can be swapped within each byte to improve tracing.

– Except Data Mask (DM[3:0]) and Data Strobe(DQS _P _N[3:0]) and bit 0 of each byte

– Gate Swapping: All members of one byte can be exchanged with all members of another byte, necessarily including Data Mask (DM[3:0]) and Data Strobe(DQS _P _N[3:0]).

All reference boards follow this rule. (send PDFs and PNGs for check)

My DNS.

D0 D8 D16 D24 on correct pins.

Engineer changes DSN:
DDR1

DDR2:

We can’t go to pcb factory with this routing, DDR will not work.

We must return in DSN bit 0 of each byte for right pin of DDR and reroute .